| Specification Section 34 75 13 TRUCKSTOPPER 5 CRASH BOLLARD |  |  |
| --- | --- | --- |

GENERAL

  - SECTION INCLUDES:

    - The work in this section shall include furnishing all labor, materials, equipment and appliances necessary to complete installation of BSI PAS68:2010 tested crash bollards required for this project in strict accordance with this specification section and drawings.

  - REFERENCES:

    - BSI PAS68:2010: Performance requirement, vehicle impact test method and performance rating.

  - SUBMITTAL:

    - Product Data:

      - Provide manufacturer’s catalog cuts with printed specifications and installation instructions.

      - Furnish detailed description of system.

    - Shop Drawings:

      - Provide shop drawings showing the bollard system, including detail describing installation technique in various surfaces (example: concrete sidewalk, asphalt, pavers, etc.).

    - Certifications:

      - Letter of compliance stating the Truckstopper 5 bollard system has successfully  passed an actural crash test and is certified to perform as described in 2.03 below (BSI PAS68:2010.

PRODUCTS

  - CRASH tested bollard MANUFACTURER:

    - The Truckstopper 5 crash bollard shall be manufactured by SafetyFlex Anti Terrorist Barriers and distributed by Tymetal Corp., 678 Wilbur Avenue, Greenwich, NY  12834 – (800) 328 – 4283.

    - Approved substitution – All other crash tested bollard systems must be submitted to the design team in accordance with substitution requirements as set forth in the general provisions of the project manual for approval prior to the bid date.  Products submitted after the bid date will not be approved.

    - Store crash bollard on building site, under cover, on wood sills or floors, and in a manner that prevents rust or damage.  Ventilate canvas or plastic covers to prevent moisture traps.

  - SYSTEM LAYOUT/DIMENSIONS:

    - Per detailed drawing.  Foundation dimensions shall remain the same should one, two or three bollards be required.

  - SYSTEM FUNCTION and CRASH RATING:

    - The Truckstopper 5 crash bollard has been designed for hostile vehicle mitigation.  It can provide perimeter security for airports, oil and gas infrastructure, military sites or any other facility requiring crash rated perimeter security.

    - The Truckstopper 5 crash bollard has been tested to the requirements of  BSI PAS68:2010 Vehicle Impact Test Method and Performance Rating.

      - Vehicle weight: 16,534 lbs. (7500 KG).

      - Vehicle speed: 50 mph (80kph)

      - Vehicle angle: 90

      - Vehicle penetration: 20 ft. (6 m)

      - Footing thickness: 16-3/4 inches (426 mm)

  - VEHICLE BARRIER:

    - Components:

      - Slipper box with locking bolts and impact plate.

      - Lock-bolted front and rear impact bars.

      - Rebar

      - Front and rear posts.

      - 316L Grade Stainless Steel Shroud

EXECUTION

  - Site INSPECTION:

    - Final grades and installation conditions shall be examined.  Installation shall not begin until all unsatisfactory conditions are corrected.

  - INSTALLATION:

    - Equipment in this section shall be installed in strict accordance with the company’s printed instructions unless otherwise shown on the contract drawings.

Tymetal Corp. reserves the right to modify and/or make changes as deemed necessary without previous notice.