| Specification Section 34 75 13 TRUCKSTOPPER 10-30 CRASH TESTED BOLLARD – SURFACE MOUNT INSTALLATION |  |  |
| --- | --- | --- |

GENERAL

  - SECTION INCLUDES:

    - The work in this section shall include furnishing all labor, materials, equipment and appliances necessary to complete installation of IWA 14 tested crash bollards required for this project in strict accordance with this specification section and drawings.

  - REFERENCES:

    - IWA 14: Performance requirement, vehicle impact test method and performance rating.

  - SUBMITTAL:

    - Product Data:

      - Provide manufacturer’s catalog cuts with printed specifications and installation instructions.

      - Furnish detailed description of system.

      - U.S. patent applies.

  - CRASH TESTED BOLLARD MANUFACTURER:

    - The Truckstopper 10-30 crash bollard shall be manufactured by SafetyFlex Anti Terrorist Barriers and distributed by Tymetal Corp., 678 Wilbur Avenue, Greenwich, NY  12834 – (800) 328 – 4283.

    - Approved substitution – All other crash tested bollard systems must be submitted to the design team in accordance with substitution requirements as set forth in the general provisions of the project manual for approval prior to the bid date.  Products submitted after the bid date will not be approved.

    - Store crash bollard on building site, under cover, on wood sills or floors, and in a manner that prevents rust or damage.  Ventilate canvas or plastic covers to prevent moisture traps.

  - SYSTEM LAYOUT/DIMENSIONS:

    - Per detailed drawing.

  - SYSTEM FUNCTION and CRASH RATING:

    - The Truckstopper 10-30 crash bollard has been designed for hostile vehicle mitigation.  It can provide perimeter security for airports, oil and gas infrastructure, military sites or any other facility requiring crash rated perimeter security.

    - The Truckstopper 10-30 crash bollard has been tested to the requirements of  IWA 14 Vehicle Impact Test Method and Performance Rating.

      - Vehicle weight:  15,873 lbs. (7200 KG).

      - Vehicle speed: 30 mph (48kph)

      - Vehicle angle: 90

      - Vehicle penetration: 26’-9” (8.2 m)

      - Surface Mount Installation

  - VEHICLE BARRIER:

    - Components:

      - Slipper box with locking bolts and impact plate.

      - Rebar

      - (4) Posts.

      - 316L Grade Stainless Steel Shroud

      - Above grade Bollard Footing:  4’-7” x 5’-11”, raised 0’ – 6 ¾” above grade

EXECUTION

  - Site INSPECTION:

    - Final grades and installation conditions shall be examined.  Installation shall not begin until all unsatisfactory conditions are corrected.

  - INSTALLATION:

    - Equipment in this section shall be installed in strict accordance with the company’s printed instructions unless otherwise shown on the contract drawings.

Tymetal Corp. reserves the right to modify and/or make changes as deemed necessary without previous notice.