| Specification Section 34 75 13 TSB-RB001-30 MANUAL RISING CRASH TESTED BOLLARD |  |  |
| --- | --- | --- |

GENERAL

  - SECTION INCLUDES:

    - The work in this section shall include furnishing all labor, materials, equipment and appliances necessary to complete installation of BSI PAS68:2010 tested crash bollards required for this project in strict accordance with this specification section and drawings.

  - REFERENCES:

    - BSI PAS68:2010: Performance requirement, vehicle impact test method and performance rating.

  - SUBMITTAL:

    - Product Data:

      - Provide manufacturer’s catalog cuts with printed specifications and installation instructions.

      - Furnish detailed description of system.

  - CRASH tested bollard MANUFACTURER:

    - The TSB-RB001-30 manual rising crash bollard system shall be manufactured by SafetyFlex Anti Terrorist Barriers and distributed by Tymetal Corp., 678 Wilbur Avenue, Greenwich, NY  12834 – (800) 328 – 4283.

    - Approved substitution – All other crash fence systems must be submitted to the design team in accordance with substitution requirements as set forth in the general provisions of the project manual for approval prior to the bid date.  Products submitted after the bid date will not be approved.

    - Store crash bollard on building site, under cover, on wood sills or floors, and in a manner that prevents rust or damage.  Ventilate canvas or plastic covers to prevent moisture traps.

  - SYSTEM LAYOUT/DIMENSIONS:

    - Per detailed drawing.

  - SYSTEM FUNCTION and CRASH RATING:

    - The TSB-RB001-30 crash bollard system has been designed for hostile vehicle mitigation.  It can provide perimeter security for airports, oil and gas infrastructure, military sites or any other facility requiring crash rated perimeter security.

    - The TSB-RB001-30 crash bollard has been tested to the requirements of  BSI PAS68:2010 Vehicle Impact Test Method and Performance Rating.

      - Vehicle weight:  16534 lbs. (7500 KG).

      - Vehicle speed: 30 mph (48kph)

      - Vehicle angle: 90

      - Vehicle penetration: 10’-5” (3.2 m)

      - Footing depth: 17-3/4” (450 mm)

  - VEHICLE BARRIER:

    - Components:

      - Bollard box assembly.

      - Impact bars.

      - Rebar

      - 316L Grade Stainless Steel Shroud

EXECUTION

  - Site INSPECTION:

    - Final grades and installation conditions shall be examined.  Installation shall not begin until all unsatisfactory conditions are corrected.

  - INSTALLATION:

    - Equipment in this section shall be installed in strict accordance with the company’s printed instructions unless otherwise shown on the contract drawings.

Tymetal Corp. reserves the right to modify and/or make changes as deemed necessary without previous notice.